MonkeyCall Issues Analyzis

# Received information

| Date | Message |
| --- | --- |
| 09.10.2023 | One collar has unfortunately not survived the journey it seems. So I am down to 32 now. I have done one test recording on one of our dogs for two days to calibrate the recording level. The equipment works without problems so far and the team is doing a superb job at darting and sample collection. Couldn´t ask for more! |
| 16.10.2023 | As we have darted most of the ‘easy’ males and have to remove old collars from females, we are opportunistic with darting at the moment. I hope to split the remaining 10 collars (one is broken) between NH-group and BD-group to have the best possible coverage. The ideal scenario would be to collar everyone in NH. The remaining males in NH and BD are, unfortunately, quite shy and don´t have old collars to remove, so we will see how many I can get. KB-group is complete and we will do the caracal presentation as soon as we get a day without rain. Only two of the collars have malfunctioned so far. Sadly, one of them was on the highest-ranking male in the AK-group, where we did the caracal presentation on Saturday. We managed to cover this individual with camera/directional microphones, however, so we do not lack a recording from him during the experiment. The caracal still works splendid at triggering barks. When I activated the recording on the day before the experiment, the collar in question did not transition into recording mode but was instead stuck in a ‘ready’ state. We will attempt the auto release of this collar tomorrow, depending on the weather. The second collar that does not work, apparently was damaged in transport as it is frequently losing connection. I have not yet found the time to take it apart to see if I can repair it. I´ll send another update when we have completed the KB-experiment and have retrieved the frozen collar in the AK-group. |
| 19.10.2023 | I have some unpleasant news. I encountered 2 new problems with 3 collars and I don´t yet know how far spread these problems are. In short, we had heavy rainfall in the last days and one collar stopped responding entirely. Unlike the previous problem, where the collar was stuck in “ready” mode, I cannot even communicate with this collar anymore for 2 days in a row now. It doesn´t show up on the app anymore. Problem 1: One collar is dead after rain. In addition, two collars that already started recording 6 days ago (in AK group) stopped recording after the rain. They switched back into the “default” mode (“hardware off”), where the only choices you have are “Reset and start recording” and “drop collar”. These two collars also show “number of days recorded: 0” instead of 6 (which would be the correct number). Lastly, the recording level switched back to 3 on these two collars (I had set it to 2). So all in all, these collars seem to have reset themselves entirely. What I do not know now, is whether I can simply activate them again (Reset and start recording) or whether this might delete the data that has been stored already. Problem 2: Two collars stopped recording and were apparently reset to default/hardware off mode. I still have signal with these two. If you could let me know, whether I can just press “Reset and start recording” on these collars without losing data that would be very helpful! I don´t know for certain that the rain has something to do with it but it appears likely, as this problem occurred one day after the heavy rain. Some good news: I have retrieved the very first problem collar that was not responding anymore (stuck in “ready mode”). The auto release worked perfectly, so no issues there! I´ll try to re-dart the male that has the non-working collar today to switch collars (I cannot even activate the auto-release on that one, since it is not showing up anymore at all). I may also retrieve the other two in order to figure out if they stored any data at all unless you are certain that I can just reactivate them without losing data. |
| 17.10.2023 | Previous answer: “He should try to click on « Toggle Record » button with the app, and then the recording should start correctly…” I did exactly that, but nothing happened. Toggle recording was not able to start it. I also turned the phone on and off and clicked on “drop collar” (clicking on “no” when its asking if you really want to drop it. I also changed recording level. So, I basically did everything I could, other than actually removing the collar, which I will do when it is not raining. It seems that this collar is stuck:/ |
| 01.11.2023 | Pour l'instant on a 6 colliers qui ont marché sans soucis. Oui ils ont pris de l'eau - mais aujourd'hui je vais démonter un des colliers qui n'ont pas marché, faire une vidéo et je t'envoie ça On a la liste papier, je t'envoie ça ce soir after meeting with Lukas |
| 02.11.2023 | some worrisome news from the field in South Africa. I think we will be able to use data coming ONLY from 30% of the collars only. I attach here a first preliminary report, but it goes without saying that this is very, very bad |
| 02.11.2023 | - Collar modalities which cause issue: - Ready mode. We cannot launch recording, it’s blocked. We can only drop. Can you do anything at the app so that we can try to record with this collar? - We activated collars this morning and rechecking again after 2h and it’s hardware off again - Bluetooth portability issues: - We have to get too close - It has max 15 meters portability which is not normal and the distance is not stable - also because below 90 RSSI we cannot transmit the command in a stable way - Euphorbia tree is shielding the signal - App displaying: - The way the remaining days is displayed is highly inefficient and it relies on the launch of the recording, which cannot be the case as we have to restart the recordings from time to time |

# Received information